The objective is to create an end-to-end prototype that can:

- select a feeder and planning horizon

- define future scenarios for EV, solar, and data center growth

- synthetically generate feeder-level load and generation profiles

- inject those profiles into an IEEE 123-bus OpenDSS feeder model

- run time-series power flow simulations

- detect overloads and voltage issues

- generate a large space of intervention portfolios

- re-simulate the feeder for each portfolio

- score and rank the portfolios

- produce engineering and business-oriented visualizations and recommendations

The prototype demonstrates how an AI-assisted or agentic planning system can support utility planners with structured scenario analysis and decision support.

Scope of the Current Prototype

The current implementation is a representative 24-hour future-day planning study.

That means:

- the simulation duration is one day with 24 hourly time steps

- the selected planning horizon does not simulate all months/years directly

- instead, the planning horizon is used to scale future load/generation magnitudes and determine whether the data center is online by that time

So, for example:

- a 12-month horizon means “simulate a representative day one year in the future”

- a 5-year horizon means “simulate a representative day five years in the future”

This is appropriate for a prototype because it keeps runtime manageable while still demonstrating scenario-based future-state planning logic.

What the Code Does

The code implements the following workflow.

1. Feeder Setup

It uses the IEEE 123-bus distribution feeder modeled in OpenDSS. The feeder includes:

- baseline circuit definition

- line data

- transformers

- regulators

- capacitors

- baseline spot loads

These are read from the OpenDSS master file and associated redirected files.

2. Scenario Definition

The user selects:

- planning horizon: 6m / 12m / 18m / 3yr / 5yr

- EV demand scenario: Low / Base / High

- solar adoption scenario: Low / Base / High

- data center demand scenario: Low / Moderate / High

- data center connection timeline: 6m / 12m / 18m

3. Synthetic Profile Generation

For each scenario, the code generates synthetic hourly profiles for:

- feeder baseline load multiplier

- EV charging demand

- solar generation

- data center load

These profiles are designed to reflect plausible operational shapes:

- feeder load: morning and evening demand structure

- EV charging: evening-heavy load pattern

- solar: bell-shaped daytime generation pattern

- data center: relatively flat large load profile

4. Feeder Simulation in OpenDSS

For each hourly time step, the code:

- compiles the IEEE 123 feeder

- scales all existing feeder baseline loads using the feeder multiplier

- adds EV loads at selected buses

- adds solar generators at selected buses

- adds data center load at a selected bus

- solves power flow using OpenDSS

5. Violation Detection

The code then extracts engineering indicators such as:

- minimum voltage

- maximum voltage

- number of undervoltage buses

- number of overvoltage buses

- number of overloaded lines

- number of overloaded transformers

- convergence failures

6. Alternative Portfolio Generation

Instead of testing only single interventions, the code generates a portfolio space of combined measures, such as:

- Transformer upgrade at 33%, 66%, or 100%

- Battery at 33%, 66%, or 100%

- Managed charging at 33%, 66%, or 100%

- Phased interconnection at 33%, 66%, or 100%

- Demand tariff at 33%, 66%, or 100%

Example portfolio:

- TransformerUpgrade = 33%

- Battery = 66%

- ManagedCharging = 33%

- PhasedInterconnection = 0%

- DemandTariff = 0%

Each portfolio is treated as a candidate planning option.

7. Re-simulation Under Each Portfolio

Each portfolio modifies the profiles and/or network capacity interpretation, then the code reruns the full 24-hour simulation and measures how much the portfolio reduces system stress.

8. Scoring and Ranking

Each portfolio is evaluated using:

- technical improvement

- cost attractiveness

- feasibility

- deployment speed

A weighted score is computed and portfolios are ranked. The output identifies:

- top recommendation

- second-best option

- top-ranked portfolio list

9. Visualization

The code also produces visual outputs such as:

- synthetic profile plots

- simulation result plots

- bus asset tables

- bus allocation maps

- breakdown/violation network maps

- before/after comparison maps

- ranked portfolio charts